List of Experiments

Academic Year: 2020-2021

Subject: EC342 (VLSI Technology & Design)	                   				        Semester: 5th

| Exp.No. | Aim of Experiment |
| --- | --- |
|  | Introduction to Xilinx ISE suite tool |
|  | Implement Half-subtractor & Full subtractor using dataflow, behavioral, structural modelling styles |
|  | - Implement and analyse the circuit shown in Figure 1 using RTL and behavioural modelling style. - Using system 1 of Task 1 as instance model, implement the model shown in Figure 2. |
|  | a. Design a full adder circuit using two half adders as instance model. Write test bench for the full Adder circuit. b. Write Verilog code to implement n-bit adder. c. Using n-bit adder as parameterized module, implement 3-bit ripple carry adder in the main module. |
|  | In navigation communication control system, the magnetic position on the compass is used to detect the direction of the ship. For detailing and sending information of the ship location to the Base ground station, the magnetic compass direction is converted into digital angular position code as shown in Fig. 1. This is performed using ‘Logic Block’ as presented in Fig. 1. The ‘Logic Block’ encodes the direction that is pointed on the magnetic compass to digital angular position code. (For ex. if North direction is chosen to be represented in digital angular position code as 3’b000, then whenever the needle on the magnetic compass points North direction, then digital angular position code should result 3’b000). Assume other directions and their digital angular position code appropriately. Write Verilog code for ‘Logic Block’ in Fig. |
|  | Generate clock using various methods. Also generate clka, clkb, clkc pulses as shown in Fig. using behavioral modelling (without using delay assignments). |
|  | (a) Design modules to swap two numbers each of 2bits using blocking and non-blocking statements without using clock signal. (b) Design modules to swap two numbers each of 2bits using blocking and non-blocking statements using clock signal. Generate clock signal of pulse period of 2 standard time units. |
|  | To implement 2-input adder. Each input is of 8-bits. The output of the adder is given to flag register. The flag resister is defined in Fig. where, status of each bits of flag register is defined as: f[0]: Zero flag: Set it as ‘1’ if output of the adder is zero, otherwise set it as ‘0’. f[1]: Parity flag: Set it as ‘1’ if odd number of ones are there in the output of adder, otherwise set it as ‘0’. f[2]: Auxiliary_carry flag: Set it as ‘1’ if carry is passed from fourth to fifth bit of the adder, otherwise set it as ‘0’. f[3]: Even_or_odd flag: Set it as ‘1’ if output of the adder is even number otherwise set it as ‘0’. f[4]: Carry flag: Set it as ‘1’ if after arithmetic operation, carry is generated, otherwise set it as ‘0’. Write HDL in Verilog to modulate appropriately the bits of flag register according to adder output. |
|  | Write a Verilog module to implement a 4-bit adder-subtractor by instantiating four fadder modules. The module will take the following arguments: i. Two 4-bit inputs A and B, ii. 1-bit carry input In, iii. 4-bit output Res, iv. 1-bit output Out if input variable In is in state 0, the module will add B to A and produce sum and carry in Res and Out output variables respectively. If the variable In is in state 1, the module will subtract B from A and store the difference and borrow in Res and Out variables respectively. |
|  | Write a Verilog module that will take as input a 16-bit DATA and produces output COUNT as the number of ‘1’s present in the input DATA string |
|  | Eight 32-bit channels D7, D6, …, D0 are to be multiplexed on a common 32-bit bus called DATABUS under the control of a 3-bit select control SEL. Write a Verilog module to implement the specification. |
|  | Write a Verilog module using switch-level modeling to implement the function F = (A.D + E.(B + C))’ using CMOS logic. You may use the nmos and pmos transistors for the purpose. |
|  | Implement task and function in Verilog. |
|  | Understand the effect of timescale in simulation results of a Verilog module. |
|  | Obtain V-I characteristics of NMOS/PMOS MOSFET using LT Spice (DC Analysis). |
|  | Calculate delay of CMOS inverter and Resistive Load Inverter using LT Spice (Transient Anaylsis) |
|  | Design NAND and NOR gate using CMOS in LT Spice. |

Mr. Yogesh Tiwari                                                                                              Dr. Trushit Upadhyaya

Lab-Incharge					             	                                   Head of Department, EC