List of Experiments

Name of Teacher: Hitesh Patel							Date: 01/06/2020

Subject: EC767 Embedded system design					             Semester: 1st (M.Tech)

| Exp. No | Aim of Experiment |
| --- | --- |
|  | Write an AVR ATMEGA16 C program to toggle all bits of PA continuously every 500 ms. Assume F = 12 MHz |
|  | A switch is connected to pin PA2. Write an AVR ATMEGA16 C program to monitor SW and create the following frequencies on pin PA1: If SW = 0 :- 500 Hz If SW = 1 :- 750 Hz |
|  | Interface 8-LED with any port of ATMEGA16 and write a C-language programs to generate different patterns on LED. |
|  | Interface 2 common anode seven segments with AVR ATMEGA16 using concept of Multiplexing. Write a C language program for following task. If push button switch connected at PB3 is pressed display down counter otherwise up counter. (counter should be 00 to 99 and time between to count should be 1 second). |
|  | Interface 4 common anode seven segments with AVR ATMEGA16 using concept of Multiplexing. Write a C language program to display digital clock (minute and second only). |
|  | Write an AVR ATMEGA16 C program to transfer serially the message “Welcome to CHARUSAT” at a 9600 baud rare (Using interrupt method). |
|  | Write an AVR ATMEGA16 C program to receive data serially with baud rate 1200 and display on LCD . |
|  | Interface 16*2 LCD with ATMEGA16. Write an AVR ATMEGA16 C program to display first name in row 1 and last name in row 2. (a) Using 8-bit mode. |
|  | Write an AVR ATMEGA16 C program to read the 4x4 keypad and send the result to the LCD |
|  | Interface LM35 with ATMEGA16. Write an AVR ATMEGA16 C program to display temperature on Serial Port. |
|  | Interface DS1307 (RTC) with ATMEGA16. Write an AVR ATMEGA16 C program to set time 10:02:06 a.m and Display clock on LCD. |
|  | Interface MAX7221 with ATMEGA16.Write an AVR ATMEGA16 C program to display 4 digit counter on 7-Segments. |