List of Experiments

Name of Teacher: Miral M. Desai							                 Date: 9/5/2020

Subject: EC344 Microcontroller & Application                                                                    Semester: 5

| Exp. No | Aim of Experiment |
| --- | --- |
|  | Interface 8-LEDs with ATMEGA16 and Write a C code to toggle all LEDs continuously after every 500 ms.(Assume Frequency = 12 MHz.) |
|  | Interface 8x8 common anode LED matrix with ATMEGA16 and Write a C code to display rotating message "WELCOME TO CHARUSAT". |
|  | Interface 8-LEDs with any port of ATMEGA16 and write a C-language programs to generate different user defined patterns on LED. SW1=0 and SW2=0 ------> pattern1 SW1=1 and SW2=0 ------> pattern2 SW1=0 and SW2=1 ------> pattern3 SW1=1 and SW2=1 ------> pattern4 |
|  | a) Interface 2 common cathode 7- segments with ATMEGA16. Write a C code for following tasks. If push button switch connected at PB0 is pressed display up counter and if push button switch connected at PB1 is pressed display down counter. If both the switches are in open condition then stop the counter(counter should be 00 to 99 and time between each count is 1 second). b) Interface 4 common cathode 7- segments with ATMEGA16. Write a C code for Stop watch with start and stop condition. |
|  | Write a C code to generate 1 KHz square wave using timer 0 in Normal mode Using interrupt. (Assume Frequency = 8 MHz) |
|  | Write a C code to generate 3 KHz square wave using timer 2 in CTC mode Using interrupt. (Assume Frequency = 8 MHz) |
|  | Assume that INT0 pin is connected to a switch that is normally high. Write a C code that toggles PC3 ,whenever INT0 pin goes low. Use the external interrupt in edge-triggered mode. |
|  | Write a C code to transfer the message “Welcome to CHARUSAT” serially at a 19200 baud rare (Using interrupt method). |
|  | Interface 16x2 LCD with ATMEGA16. Write a C program to receive data serially with baud rate 4800 and display on LCD in both row. |
|  | Interface 20x4 LCD and I2C based LCD controller with ATMEGA16. Write an C code to display following message using inbuilt TWI (Two wire) protocol of Atmega16 . "Welcome To" in first row "The World of" in second row "Embedded" in third row "System" in fourth row |
|  | Interface 4x3 keypad and External I2C based EEPROM and LCD with ATMEGA16. Write a C code to store password in EEPROM and read the password from user through keypad and display message on LCD "Correct password " or "Wrong password". |
|  | Interface LM35 and 3 common cathode 7-segments with ATMEGA16. Write a C code to display temperature on Seven segment. |
|  | Interface DS1307 (RTC) ,16x2 LCD and I2C based LCD controller with ATMEGA16. Write a C code to set current time in 12 hour mode and date in RTC and display time in first row and date in second row. |
|  | Interface MAX7221 SPI based 7-segment controller and 8-digit common anode seven segment with ATMEGA16.Write an AVR ATMEGA16 C program to display message "UP" on first 2 segments and 4 digit up counter on last 4 Segments. |