CHAROTAR UNIVERSITY OF SCIENCE & TECHNOLOGY

Fifth Semester of B. Tech (EC) Examination

NOV-DEC 2018

EC 344 Microcontroller & Application

Date: 14.11.2019, Thursday		Time: 10.00 a.m. To 01.00 p.m.	Maximum Marks: 70

Instructions:

- The question paper comprises two sections.

- Section I and II must be attempted in separate answer sheets.

- Make suitable assumptions and draw neat figures wherever required.

- Use of scientific calculator is allowed.

- Registers are given on last page of question paper.

| SECTION – I |  |  |  |
| --- | --- | --- | --- |
| Q-1 | Do as Directed. |  | 07 |
|  |  | TIMSK register is used for? i) knowing the status of the timer count ii) used for masking the interrupts flags of the Timer0, Timer1 and Timer2 iii) it is used for enabling all the timer interrupts iv) it is used for resetting the value of the interrupts |  |
|  |  | True or False. An ATMEGA32 AVR is 16-bit Microcontroller. |  |
|  |  | What is the use of the prescalar in the operation of the timer? i) for fast calculations ii) for increasing the time delay given by the timer by decreasing its frequency of operation iii) for removing the concept the reloading of count iv) for easy counter operations |  |
|  |  | Which of the timer can operate in the 16 bit condition? i) timer0 ii) timer1 iii) timer2 iv) all of the mentioned |  |
|  |  | With Fosc=8 MHz, what will the count that has to filled in the UBRR register to account for the 9600 baud rate? i) 67H ii) CEH iii) 33H iv) 34H |  |
|  |  | Which of the following will generate the maximum time delay? i) f/2 ii) f/4 iii) f/16 iv) f/32 |  |
|  |  | In an ADC, we can calculate the output voltage from the formula i) Step size / Vin ii) Step size * Vin iii) Vin / step size iv) Vref / step size |  |
| Q-2(a) | Interface 16x2 LCD with ATMEGA16. Write an C code to display following message "Winter" in first row & "2019" in second row |  | 07 |
| (b) | Write an AVR C program to toggle all the pins of PORTC continuously with a 10ms delay. Use a predefined delay function. |  | 04 |
|  | OR |  |  |
| (b) | Write an AVR C program to get byte of data from PORTC. if it is less than 100,send it to PORTB; otherwise send it to PORTD. |  | 04 |
| (c) | A door sensor is connected to bit 1 of PORTB, and an led is connected to bit 7 of PORTC. Write an AVR C program to monitor the door sensor and, when it opens, turn on the led. |  | 03 |
| Q-3 | Answer the following Questions. (Any Two) |  | 14 |
| (a) | Interface 8-LEDs with PORTA of ATMEGA16 and Write an AVR C code to toggle all LEDs continuously after every 200ms.(Use Timer 0 for delay and Assume Fosc = 1 MHz and Prescalar=1024). |  |  |
| (b) | Write an AVR C program to write “welcome to CHARUSAT” string on internal EEPROM and read back the same and display on serial monitor using UART. |  |  |
| (c) | Interface MAX7221 SPI based 7-segment controller and 8-digit common anode seven segments with ATMEGA16. Write a C program to display message "UP" on first 2 segments and 2 digit up counter on last 2 Segments. |  |  |
| SECTION – II |  |  |  |
| Q-4 | Interface 2 common cathode 7- segments with ATMEGA16. Write a C code for the following task. If push button switch connected at PB0 is pressed display up counter and if push button switch connected at PB1 is pressed display down counter. If both the switches are in open condition then stop the counter (counter should be 00 to 99 and time between each count is 1 second). |  | 07 |
| Q-5 | Answer the following Questions. (Any Two) |  | 14 |
| (a) | Write a C code to generate Fast PWM with 40% duty cycle using timer 0 and find out the frequency of PWM. (Assume Fosc = 1 MHz and prescalar=1024). |  |  |
| (b) | Assume that INT0 pin is connected to a switch that is normally high. Write a C code that toggles PC3, whenever INT0 pin goes low. Use the external interrupt in edge-triggered mode. |  |  |
| (c) | Interface LM35 and 16x2 LCD with ATMEGA16. Write a C code to display temperature on LCD. |  |  |
| Q-6 | Answer the following Questions. (Any Two) |  | 14 |
| (a) | Write a C code to generate 1 KHz square wave using timer 0 in Normal mode Using interrupt. (Assume Frequency = 8 MHz). |  |  |
| (b) | Write a C code to transfer the message “Embedded System” serially at a 9600 baud rare (Using interrupt method). |  |  |
| (c) | Interface 4x4 Keypad with ATMEGA16. Write an AVR C Program to detect the key pressed by user and send it on PORTD. |  |  |

TCCR0

TCCR2

GICR

MCUCR

UCSRA

UCSRB

UCSRC

SPCR

SPSR

ADMUX

ADCSRA